CS 350 – Milestone Two Report

GPIO and UART Serial Communication

Russell Buck

Southern New Hampshire University

1. Lab Questions

1. Why do both the SerialTest-Write.py and SerialLightControl-Client.py scripts use the encode() method when writing data to the serial port?

Both scripts use the encode() method because serial communication hardware transmits bytes, not Python string objects. Python strings are Unicode by default, so they must be converted into a byte sequence before being sent over UART. The encode() method performs this conversion, typically using UTF-8 encoding. Without encoding, the serial driver would not be able to transmit the data, since UART interfaces operate strictly on raw byte values.

2. Why does the SerialTest-Read.py script use the decode() method when reading the data from the serial port?

When reading from the serial port, the data arrives as raw bytes. To interpret these bytes as human-readable text or commands (such as "on", "off", "exit", or "quit"), the program must convert the byte sequence back into a Python string. The decode() method performs this conversion using UTF-8 decoding. This allows the program to compare the received input against expected command strings and respond appropriately.

3. What is the purpose of the try/except block in both the SerialLightControl-Client.py script and the SerialLightControl-Server.py script?

The try/except block ensures that the program can handle unexpected interruptions—especially KeyboardInterrupt (CTRL-C)—without leaving the system in an unsafe or inconsistent state. In embedded systems, it is critical to release hardware resources properly. The except block allows the program to:

- Turn off the LED

- Reset GPIO pins to their default state

- Close the serial connection

- Exit cleanly

This prevents hardware conflicts, protects the Raspberry Pi’s GPIO subsystem, and ensures the next program can run without errors.

4. Why is it necessary to make sure that the GPIO pins are always returned to their original state at the end of program run?

GPIO pins control real electrical signals. Leaving them active after a program ends can cause unintended behavior, such as LEDs remaining lit, components overheating, or electrical conflicts with future programs. Resetting the pins ensures:

- The LED and other components are safely powered down

- No voltage remains on output pins

- The GPIO subsystem is returned to a known, stable state

- Future scripts can configure the pins without errors

Calling GPIO.cleanup() is a best practice in all Raspberry Pi applications to maintain hardware safety and reliability.

2. State-Machine Explanation

The SerialLightControl system operates as a finite state machine that manages LED behavior based on serial commands received from a client. The machine begins in the Idle state, where it waits for input from the serial port. When the command “on” is received, the system transitions to the ON state, activating GPIO 18 and illuminating the LED. When the command “off” is received, the system transitions to the OFF state, turning the LED off by disabling output on GPIO 18. From any state—Idle, ON, or OFF—the system can receive either “exit” or “quit”, which triggers a transition to the Exit state. In this final state, the program ensures the LED is turned off, performs GPIO cleanup to restore all pins to their default configuration, and terminates execution cleanly. This state-machine design provides a clear, deterministic control flow that mirrors the serial communication logic implemented in the Python server script.

3. Conclusion

This milestone demonstrates how UART serial communication and GPIO control can be integrated to create a responsive embedded system. By configuring the Raspberry Pi’s serial interface, implementing a state-machine-driven server, and validating communication through a client script, the system reliably responds to external commands. Proper cleanup and error handling ensure safe and predictable hardware behavior, which is essential in embedded development.